Concentration polarization.

We begin with the Nernst Planck equation and assume that diffusion, convection and electromigration are the dominant transport mechanisms (Eq 1) of ions in solution. Here ji is the flux of an ion, D if the diffusion coefficient of species ‘i’ in solution, ∇c is the concentration gradient of species ‘i’. This analysis is for concentration polarization, as such the gradient extends from the membrane solution interface to the bulk concentration. v is the velocity of solvent, e is the elementary charge, kB is the Boltzmann constant, T is temperature and E is the electric field.  ∇ϕ is the electrical potential from the bulk solution to the interface. Additionally, without an external potential, the net ionic current through the surface of the membrane is equal to 0 (Eq 2).

| ji= -Di∇ci+civ-DiziekBTciE ji= -Di∇ci+ciJw-ziDiFRTci∇ϕ | (1) |
| --- | --- |
| I=0=SinziFji dr=0 I=0= Sinziji dr | (2) |

Equation (1) and (2) can be used to determine the electrical potential Equation (3) for a single salt. z1 and z2 are the charges of ion species 1 and 2, respectively. In this case, species 1 will refer to the cation, while species 2 will refer to the anion.

| 0= -D1z1∇c1+z1c1Jw-D1z12FRTc1∇ϕ-D2z2∇c2+z2c2Jw-D2z22FRTc2∇ϕ D1z12FRTc1∇ϕ+D2z22FRTc2∇ϕ= -D1z1∇c1+z1c1Jw-D2z2∇c2+z2c2Jw ∇ϕD1z12FRTc1+D2z22FRTc2=-D1z1∇c1+z1c1Jw-D2z2∇c2+z2c2Jw ∇ϕ=RTF -D1z1∇c1+z1c1Jw-D2z2∇c2+z2c2JwD1z12c1+D2z22c2 | (3) |
| --- | --- |

The result can be simplified (Equation 5) when we account for electroneutrality within the solution, Equation (4).

| inzici=0 z1c1+z2c2=0 c1= -z2z1c2, c2= -z1z2c1 Similarly, take the derivative/gradient. z1∇c1+z2∇c2=0 ∇c1=-z2z1∇c2, ∇c2=-z1z2∇c1 ∇ϕ=RTF -D1z1∇c1-D2z2∇c2+Jwz1c1+z2(-z1z2c1)D1z12c1+D2z22c2 ∇ϕ=RTF -D1z1∇c1-D2z2∇c2D1z12c1+D2z22c2 ∇ϕ=-RTF D1z1∇c1+D2z2∇c2D1z12c1+D2z22c2 | (4) |
| --- | --- |

Substituting the final result of Eq 5 back into Eq 1 yields. The ion charges, zi, should be in absolute values.

| ji=j1= -D1∇c1+c1Jw-z1D1FRTc1-RTF D1z1∇c1+D2z2∇c2D1z12c1+D2z22c2 j1= -D1∇c1+c1Jw+z1D1c1 D1z1∇c1+D2z2∇c2D1z12c1+D2z22c2 j1= -D1∇c1+c1Jw+z1D1c1 D1z1∇c1+D2z2-z1z2∇c1D1z12c1+D2z22-z1z2c1 j1= -D1∇c1+c1Jw+z1D1c1 D1z1∇c1-D2z1∇c1D1z12c1-D2z1z2c1 j1= -D1∇c1+c1Jw+z1D1c1 z1∇c1D1-D2D1z12c1-D2z1z2c1 j1=c1Jw-D1∇c1D1z12c1-D2z1z2c1D1z12c1-D2z1z2c1+z1D1c1 z1∇c1D1-D2D1z12c1-D2z1z2c1 j1=c1Jw-D12z12c1∇c1-D1D2z1z2c1∇c1-D1D2z2∇c1χz12D1c1-z1z2D2c1-z2D2χ+ z12D1c1∇c1D1-D2z12D1c1-z1z2D2c1-z2D2χ j1=c1Jw+-z12D12c1∇c1+z1z2D1D2c1∇c1+z12D12c1∇c1-z12D1D2c1∇c1z12D1c1-z1z2D2c1 j1= c1Jw + z1z2D1D2c1∇c1-z12D1D2c1∇c1z12D1c1-z1z2D2c1 j1= c1Jw + ∇c1z1z2D1D2c1-z12D1D2c1z12D1c1-z1z2D2c1 j1= c1Jw + ∇c1c1z1z2D1D2-z12D1D2z12D1c1-z1z2D2c1 j1= c1Jw + c1z1z2D1D2-z12D1D2z12D1c1-z1z2D2c1∇c1 j1= c1Jw + z1z2D1D2c1-z12D1D2c1z12D1c1-z1z2D2c1∇c1 j1= c1Jw + z2D1D2-z1D1D2z1D1-z2D2∇c1 j1= c1Jw + D1D2(z2-z1)z1D1-z2D2∇c1 This result is consistent with what is found in the literature. | (5) |
| --- | --- |

The concentration gradient of the polarization layer can be obtained by realizing that at the solution-membrane interface (i.e., the boundary layer), the flux of solute is equal to the flux of the ion through the membrane.

| Jwcp,1=c1Jw-D1D2¦z1¦-¦z2¦D1¦z1¦-D2¦z2¦dc1dx D12= D1D2¦z1¦-¦z2¦D1¦z1¦-D2¦z2¦ Jwcp,1=c1Jw-D12dc1dx expJw,1δD12=ci,1-cp,1cb,1-cp,1 | (6) |
| --- | --- |

*Can be performed in terms of ci,2 as well.

-------------------------------------------------------------------------------------------------------------------------------

We repeat the above derivation (i.e., concentration polarization) generalizing to three ions. Equations (1) and (2) will both remain true as they were kept in their most general form.

| ji= -Di∇ci+civ-DiziekBTciE ji= -Di∇ci+ciJw-ziDiFRTci∇ϕ | (1) |
| --- | --- |
| I=0=SinziFji dr=0 I=0= Sinziji dr | (2) |

For a three-salt system in which two cations (subscripts 1, 2) share a common anion (subscript 3), the electrical potential can be written by setting equations (1) and (2) equal to one another. (Equation M3).

| ji= 0= -z1D1∇c1+z1c1Jw-z12D1FRTc1∇ϕ-z2D2∇c2+z2c2Jw-z22D2FRTc2∇ϕ-z3D3∇c3+z3c3Jw-z32D3FRTc3∇ϕ 0=-z1D1∇c1-z2D2∇c2-z3D3∇c3+Jwz1c1+z2c2+z3c3-FRT∇ϕz12D1c1+z22D2c2+z32D3c3 -z1D1∇c1-z2D2∇c2-z3D3∇c3=FRT∇ϕz12D1c1+z22D2c2+z32D3c3 ∇ϕ=-RTFz1D1∇c1+z2D2∇c2+z3D3∇c3z12D1c1+z22D2c2+z32D3c3 | (M3) |
| --- | --- |

Electroneutrality for the three-ion system we are interested in (i.e., two cations (1,2) with a common anion (3)).

| inzici=0 z1c1+z2c2-z3c3=0 c1=-z2z1c2-z3z1c3 ;c2=-z1z2c1-z3z2c3 ; c3= -z1z3c1-z2z3c2 z1∇c1+z2∇c2+z3∇c3=0 ∇c1=-z2z1∇c2-z3z1∇c3 ; ∇c2=-z1z2∇c1-z3z2∇c3 ; ∇c3= -z1z3∇c1-z2z3∇c2 | (M4) |
| --- | --- |

With the equations derived above, we can solve for the individual ion fluxes in solution. Equation (1) is brought down to show the governing equation. The ion charges, zi, should be in absolute values.

| ji= -Di∇ci+ciJw-ziDiFRTci∇ϕ | (1) |
| --- | --- |
| j1= -D1∇c1+c1Jw-z1D1FRTc1-RTFz1D1∇c1+z2D2∇c2+z3D3∇c3z12D1c1+z22D2c2+z32D3c3 j1= -D1∇c1+c1Jw+z1D1c1z1D1∇c1+z2D2∇c2+z3D3∇c3z12D1c1+z22D2c2+z32D3c3 j1= -D1∇c1+c1Jw+z1D1c1z1D1∇c1+z2D2∇c2+z3 D3-z1z3∇c1-z2z3∇c2z12D1c1+z22D2c2+z32D3-z1z3c1-z2z3c2 j1= -D1∇c1+c1Jw+z1D1c1z1D1∇c1+z2D2∇c2-z1 D3∇c1-z2 D3∇c2z12D1c1+z22D2c2-z1z3D3c1-z2z3D3c2 We want to put this equation into the form, j1=c1Jw+"D11"∇c1+"D12"∇c2. To start doing this, we multiply the first term by “1”, which we do by taking the denominator of the fraction. j1= c1Jw-D1∇c1z12D1c1+z22D2c2-z1z3D3c1-z2z3D3c2c1z12D1-z1z3D3+c2z22D2-z2z3D3+z1D1c1z1D1∇c1+z2D2∇c2-z1 D3∇c1-z2 D3∇c2c1z12D1-z1z3D3+c2z22D2-z2z3D3 j1= c1Jw-z12D12c1∇c1+z22D1D2c2∇c1-z1z3D1D3c1∇c1-z2z3D1D3c2∇c1c1z12D1-z1z3D3+c2z22D2-z2z3D3+z12D12c1∇c1+z1z2D1D2∇c2c1-z12D1D3∇c1c1-z1z2 D1D3∇c2c1c1z12D1-z1z3D3+c2z22D2-z2z3D3 j1= c1Jw+-z12D12c1∇c1-z22D1D2c2∇c1+z1z3D1D3c1∇c1+z2z3D1D3c2∇c1+z12D12c1∇c1+z1z2D1D2∇c2c1-z12D1D3∇c1c1-z1z2 D1D3∇c2c1c1z12D1-z1z3D3+c2z22D2-z2z3D3 j1= c1Jw+-z22D1D2c2∇c1+z1z3D1D3c1∇c1+z2z3D1D3c2∇c1+z1z2D1D2∇c2c1-z12D1D3∇c1c1-z1z2 D1D3∇c2c1c1z12D1-z1z3D3+c2z22D2-z2z3D3 j1= c1Jw+∇c1z1z3D1D3c1-z22D1D2c2+z2z3D1D3c2-z12D1D3c1+∇c2z1z2D1D2c1-z1z2 D1D3c1c1z12D1-z1z3D3+c2z22D2-z2z3D3 j1= c1Jw+∇c1z1z3D1D3c1-z22D1D2c2+z2z3D1D3c2-z12D1D3c1c1z12D1-z1z3D3+c2z22D2-z2z3D3+∇c2z1z2D1D2c1-z1z2 D1D3c1c1z12D1-z1z3D3+c2z22D2-z2z3D3 j1= c1Jw+D11∇c1+D12∇c2 D11=z1z3D1D3c1-z22D1D2c2+z2z3D1D3c2-z12D1D3c1c1z12D1-z1z3D3+c2z22D2-z2z3D3 D12=z1z2D1D2c1-z1z2 D1D3c1c1z12D1-z1z3D3+c2z22D2-z2z3D3 |  |

We can repeat this process for j2

| ji= -Di∇ci+ciJw-ziDiFRTci∇ϕ | (1) |
| --- | --- |
| j2= -D2∇c2+c2Jw-z2D2FRTc2-RTFz1D1∇c1+z2D2∇c2+z3D3∇c3z12D1c1+z22D2c2+z32D3c3 In a similar manner to what was previously shown for j1, this equation will be simplified into the following form. j2= c2Jw-D21∇c1+D22∇c2 j2= -D2∇c2+c2Jw+z2D2c2z1D1∇c1+z2D2∇c2+z3D3∇c3z12D1c1+z22D2c2+z32D3c3 j2= -D2∇c2+c2Jw+z2D2c2z1D1∇c1+z2D2∇c2+z3D3-z1z3∇c1-z2z3∇c2z12D1c1+z22D2c2+z32D3-z1z3c1-z2z3c2 j2= c2Jw-D2∇c2+z2D2c2z1D1∇c1+z2D2∇c2-z1D3∇c1-z2D3∇c2z12D1c1+z22D2c2-z1z3D3c1-z2z3D3c2 j2= c2Jw-D2∇c2z12D1c1+z22D2c2-z1z3D3c1-z2z3D3c2c1z12D1-z1z3D3+c2z22D2-z2z3D3+z1z2D1D2c2∇c1+z22D22c2∇c2-z1z2D2D3c2∇c1-z22D2D3c2∇c2c1z12D1-z1z3D3+c2z22D2-z2z3D3 j2= c2Jw+-z12D1D2c1∇c2-z22D22c2∇c2+z1z3D2D3c1∇c2+z2z3D2D3c2∇c2c1z12D1-z1z3D3+c2z22D2-z2z3D3+z1z2D1D2c2∇c1+z22D22c2∇c2-z1z2D2D3c2∇c1-z22D2D3c2∇c2c1z12D1-z1z3D3+c2z22D2-z2z3D3 j2= c2Jw+-z12D1D2c1∇c2-z22D22c2∇c2+z1z3D2D3c1∇c2+z2z3D2D3c2∇c2+z1z2D1D2c2∇c1+z22D22c2∇c2-z1z2D2D3c2∇c1-z22D2D3c2∇c2c1z12D1-z1z3D3+c2z22D2-z2z3D3 j2= c2Jw+-z12D1D2c1∇c2+z1z3D2D3c1∇c2+z2z3D2D3c2∇c2+z1z2D1D2c2∇c1-z1z2D2D3c2∇c1-z22D2D3c2∇c2c1z12D1-z1z3D3+c2z22D2-z2z3D3 j2= c2Jw+ ∇c2z1z3D2D3c1-z12D1D2c1+z2z3D2D3c2-z22D2D3c2+∇c1z1z2D1D2c2-z1z2D2D3c2c1z12D1-z1z3D3+c2z22D2-z2z3D3 j2= c2Jw+∇c1z1z2D1D2c2-z1z2D2D3c2c1z12D1-z1z3D3+c2z22D2-z2z3D3+∇c2z1z3D2D3c1-z12D1D2c1+z2z3D2D3c2-z22D2D3c2c1z12D1-z1z3D3+c2z22D2-z2z3D3 Overall, the flux for ion 2 can be written as j2=c2Jw+D21∇c1+D22∇c2 D21=c2z1z2D1D2-z1z2D2D3c1z12D1-z1z3D3+c2z22D2-z2z3D3 D22=c1z1z3D2D3-z12D1D2+c2z2z3D2D3-z22D2D3c1z12D1-z1z3D3+c2z22D2-z2z3D3 |  |

We can repeat this process for j3

| ji= -Di∇ci+ciJw-ziDiFRTci∇ϕ | (1) |
| --- | --- |
| j3= -D3∇c3+c3Jw-z3D3FRTc3-RTFz1D1∇c1+z2D2∇c2+z3D3∇c3z12D1c1+z22D2c2+z32D3c3 In a similar manner to what was previously shown for j1 and j2, this equation will be simplified into the following form. j3= c3Jw-D31∇c1+D32∇c2 The simplification will not be shown. Rather, the final equation and diffusion terms are written as, j3=(-z1z3c1-z2z3c2)Jw-D31∇c1+D32∇c2 D31=c1z13z3D1D2-z12D32+c2(z1z22z3D2D3-z1z2D1D3)c1(z12D1-z1z3D3)+c2(z22D2-z2z3D3) D32=c1z12z2z3D1D3-z1z2D32+c2(z23z3D2D3-z22D2D3)c1(z12D1-z1z3D3)+c2(z22D2-z2z3D3) However, j3 is better related through the net-ionic current = 0 constraint I=0=SinziFji dr=0 0= z1j1+z2j2+z3j3 z1j1+z2j2=-z3j3 You can then plug into the equations for j1 and j2 and solve for j3. |  |

Again, the concentration gradient of the polarization layer can be obtained by realizing that at the solution-membrane interface (i.e., the boundary layer), the flux of solute is equal to the flux of the ion through the membrane.

| Jwcp,1= j1 | (M6) |
| --- | --- |

Donnan Exclusion: Multicomponent

The chemical potential of species, i, in the membrane is equal to the chemical potential of the species in the solution. Here, the solutions are assumed to be ideal, and the activity coefficient is reduced to 1.

| μim=μis μi0+RTlnγicim+ziFϕm=μi0+RTlnγicis+ziFϕs μi0+RTlncim+ziFϕm=μi0+RTlncis+ziFϕs ϕm-ϕs=RTziFlnciscim |  |
| --- | --- |

The electrostatic force of each ion is equal once the valance of the ion is accounted for.  Generally, for counterions 1 and 2 and co-ions, 3 the following equation result. Here, it is assumed that the membrane is negatively charged (e.g., NF270 membranes). This means the counterions are Na+, Ca2+, La3+ and the co-ions are Cl-.

| RTz1Flnc1sc1m=RTz2Flnc2sc2m =-RTz3Flnc3sc3m |  |
| --- | --- |
| 1z1lnc1sc1m=1z2lnc2sc2m =-1z3lnc3sc3m |  |
| lnc1sc1m1z1=lnc2sc2m1z2=lnc3mc3s1z3 |  |
| c1sc1m1z1=c2sc2m1z2=c3mc3s1z3 |  |

Electroneutrality constraint must be maintained both within the solution and within the membrane. Here, χ is the charge density of the membrane.

| i=1nzici=0 |  |
| --- | --- |
| z1c1s+z2c2s=z3c3s |  |
| z1c1m+z2c2m=χ+z3c3m |  |

Determining the concentration profile for each species in the membrane.

| c1sc1m1z1=c3mc3s1z3 c1s1z1c1m1z1=c3m1z3c3s1z3 c1s1z1c3s1z3=c1m1z1c3m1z3 c1s1z1z1z3c1s+z2z3c2s1z3=c1m1z1z1z3c1m+z2z3c2m-χz31z3 … recall c1sc1m1z1=c2sc2m1z2 c2m=c2sc1sc1mz2z1 =c2sc1mz2z1c1sz2z1 …Substitute in for c2m c1s1z1z1z3c1s+z2z3c2s1z3=c1m1z1z1z3c1m+z2z3c2sc1mz2z1c1sz2z1 -χz31z3 c1s1z1z1z3c1s+z2z3c2s1z3=c1m1z1z1z3c1m+z2z3c2sc1mc1sz2z1-χz31z3 …while complicated, this equation describes the concentration of solute 1 in the membrane as a function of the concentration of solute 1 and 2 in the solution and the charge density of the membrane. … The concentration of solute 1 in the membrane can be determined using a solver. |  |
| --- | --- |
| … we can also need to relate the concentration of solute 2 in the membrane to the solution composition. c2sc2m1z2=c3mc3s1z3 c2s1z2c2m1z2=c3m1z3c3s1z3 c2s1z2c3s1z3=c2m1z2c3m1z3 c2s1z2z1z3c1s+z2z3c2s1z3=c2m1z2z1z3c1m+z2z3c2m-χz31z3 … recall c1sc1m1z1=c2sc2m1z2 c1m=c1sc2sc2mz1z2 =c1sc2mz1z2c2sz1z2 …Substitute in for c1m c2s1z2z1z3c1s+z2z3c2s1z3=c2m1z2z1z3c1sc2mz1z2c2sz1z2+z2z3c2m-χz31z3 c2s1z2z1z3c1s+z2z3c2s1z3=c2m1z2z1z3c1sc2mc2sz1z2+z2z3c2m-χz31z3 …while complicated, this equation describes the concentration of solute 2 in the membrane as a function of the concentration of solute 1 and 2 in the solution and the charge density of the membrane. … The concentration of solute 1 in the membrane can be determined using a solver. |  |
| … we can also relate the concentration of solute 3 in the membrane to the solution composition. c2sc2m1z2=c3mc3s1z3 c2s1z2c2m1z2=c3m1z3c3s1z3 c2s1z2c3s1z3=c2m1z2c3m1z3 z3z2c3s-z1z2c1s1z2c3s1z3=χz2+z3z2c3m-z1z2c1m1z2c3m1z3 … recall c1sc1m1z1=c3mc3s1z3 c1m=c1sc3mc3sz1z3= c1s c3sz1z3 c3mz1z3= …Substitute in for c1m z3z2c3s-z1z2c1s1z2c3s1z3=χz2+z3z2c3m-z1z2c1s c3sz1z3 c3mz1z31z2c3m1z3 z3z2c3s-z1z2c1s1z2c3s1z3=χz2+z3z2c3m-z1z2c1sc3sc3mz1z31z2c3m1z3 …while complicated, this equation describes the concentration of solute 2 in the membrane as a function of the concentration of solute 1 and 2 in the solution and the charge density of the membrane. … The concentration of solute 1 in the membrane can be determined using a solver. |  |

Chang, physiochemical hydrodynamics probestein