Figure S1 – adapted from permeate probe manuscript: depicting the bulk retentate, boundary layer, membrane, permeate bulk. cpLs is approximately equal to cpint (show calculation to prove this – cite permeate probe manuscript).

*Note: the equations in green are those that will be included in the main text.

Nomenclature:

- Derivation of theoretical equations for solution and membrane phase transport.

- Solution Phase Transport: General solute flux equation through the boundary layer.

In the following derivations we look at a two-salt system in which the cations (z1, z2) share a common anion (z3). We begin with the extended Nernst Planck equation (equation 1) which assumes that diffusion, convection and electromigration are the dominant ion transport mechanisms in solution. ji is the ion flux, Di and ∇ci are the diffusion coefficient and concentration gradient of species ‘i’ in solution, Jw is the solvent flux (water), T is temperature, R is the gas constant, F is Faraday’s constant, and ∇ϕ is the electrical potential in the bulk solution.

| jis= -Dis∇cis+cisJw-ziDisFRTcis∇ϕ | Eq 1 |
| --- | --- |

There is no external potential therefor the net ionic current (I) in solution is equal to 0 (equation 2). Furthermore, the electroneutrality constraint must be upheld (equation 3).

| I=0= Sinzijis dr | Eq 2 |
| --- | --- |

| inzicis=0 | Eq 3 |
| --- | --- |

| z1c1s+z2c2s+ z3c3s=0 | Eq 4 |
| --- | --- |

| inzi∇cis=0 | Eq 5 |
| --- | --- |

| z1∇c1s+z2∇c2s+z3∇c3s=0 | Eq 6 |
| --- | --- |

Next, we solve for the electrical potential by setting equation 1 equal to equation 2.

| j1s= 0=z1c1sJw-z1D1s∇c1s-z12D1sFRTc1s∇ϕ+z2c2sJw-z2D2s∇c2s-z22D2sFRTc2s∇ϕ+z3c3sJw-z3D3s∇c3s+z32D3sFRTc3s∇ϕ | Eq 7 |
| --- | --- |

| 0=Jwz1c1s+z2c2s+z3c3s-z1D1s∇c1s-z2D2s∇c2s-z3D3s∇c3s+-FRT∇ϕz12D1sc1s+z22D2sc2s+z32D3sc3s | Eq 8 |
| --- | --- |

| FRT∇ϕz12D1sc1s+z22D2sc2s+z32D3sc3s=Jwz1c1s+z2c2s+z3c3s-z1D1s∇c1s-z2D2s∇c2s-z3D3s∇c3s | Eq 9 |
| --- | --- |

| ∇ϕ=RTF[Jwz1c1s+z2c2s+z3c3s-z1D1s∇c1s-z2D2s∇c2s-z3D3s∇c3sz12D1sc1s+z22D2sc2s+z32D3sc3s] | Eq 10 |
| --- | --- |

We apply the electroneutrality constraint (equation 4, 6) to equation 10.

| ∇ϕ=RTF[Jw0-[z1D1s∇c1s+z2D2s∇c2s+D3s(-z1∇c1s-z2∇c2s)]z12D1sc1s+z22D2sc2s+z3D3s(-z1c1s-z2c2s)] | Eq 11 |
| --- | --- |

After simplification,

| ∇ϕ=-RTF[∇c1sz1D1s-z1D3s+∇c2sz2D2s-z2D3s c1s z12D1s-z1z3D3s+ c2sz22D2s-z2z3D3s] | Eq 12 |
| --- | --- |

We can now solve for the individual ion fluxes in the solution phase. Substituting equation 12 into equation 1, we solve for the flux of cation 1 in the solution.

| j1s=c1sJw-D1s∇c1s-z1D1sFRTc1s-RTF[∇c1sz1D1s-z1D3s+∇c2sz2D2s-z2D3s c1s z12D1s-z1z3D3s+ c2sz22D2s-z2z3D3s] | Eq 13 |
| --- | --- |

| j1s=c1sJw-D1s∇c1s+z1D1sc1s∇c1sz1D1s-z1D3s+∇c2sz2D2s-z2D3s c1s z12D1s-z1z3D3s+ c2sz22D2s-z2z3D3s | Eq 14 |
| --- | --- |

We aim to put the above solution phase solute flux equation in the form j1s=c1sJw+D11s∇c1s+D12s∇c2s. Equation 14 is mathematically equivalent to equation 15.

| j1s=c1sJw-D1s∇c1sc1s z12D1s-z1z3D3s+ c2sz22D2s-z2z3D3sc1s z12D1s-z1z3D3s+ c2sz22D2s-z2z3D3s+z1D1sc1s∇c1sz1D1s-z1D3s+∇c2sz2D2s-z2D3s c1s z12D1s-z1z3D3s+ c2sz22D2s-z2z3D3s | Eq 15 |
| --- | --- |

Expanding each term and separating the variables such that ∇c1s and ∇c2s are isolated leads to equation 16,

| j1s=c1sJw+∇c1sc1s z1z3D1sD3s-z12D1sD3s+c2sz2z3D1sD3s-z22D1sD2sc1s z12D1s-z1z3D3s+ c2sz22D2s-z2z3D3s+∇c2sc1s z1z2D1sD2s-z1z2D1sD3sc1s z12D1s-z1z3D3s+ c2sz22D2s-z2z3D3s | Eq 16 |
| --- | --- |

Where the diffusion terms, D11s and D12s, are represented in the parenthesis.

We repeat this process to solve for the ion flux of cation 2, giving equation 17 in the form of j2s=c2sJw+D21s∇c1s+D22s∇c2s.

| j2s=c2sJw+∇c1sc2s z1z2D1sD2s-z1z2D2sD3sc1s z12D1s-z1z3D3s+ c2sz22D2s-z2z3D3s+∇c2sc1s z1z3D2sD3s-z12D1sD2s+c2sz2z3D2sD3s-z22D2sD3sc1s z12D1s-z1z3D3s+ c2sz22D2s-z2z3D3s | Eq 17 |
| --- | --- |

Again, where the diffusion terms, D21s and D22s, are represented in the parenthesis.

For j3s=c3sJw+D31s∇c1s+D32s∇c2s, the above derivation is repeated. Alternatively, the net-ionic current constraint can be used. Recall equation 2,

| I=0= Sinzijis dr | Eq 2 |
| --- | --- |
| z1j1s+z2j2s+z3j3s=0 | Eq 18 |

| j3s=-z1z3j1s-z2z3j2s | Eq 19 |
| --- | --- |

    - Cross comparison with literature.

Reassuringly, in the limit that we only have a one salt system, the above equations collapse to those previously derived throughout literature. Here, the cation is z1 and the anion is z3.  To illustrate this, in the limit that the concentration of cation 2 approaches 0, Equation 16 simplifies to equation 20.

| j1s=c1sJw-D11s∇c1s | Eq 20 |
| --- | --- |

| z1c1s+z3c3s=0 | Eq 21 |
| --- | --- |

| j1s=c1sJw+∇c1sD1sD3s z3-z1z1D1s-z3D3s | Eq 22 |
| --- | --- |

    - Significance of the derived equations: Concentration polarization

Equations 16, 17, and 19 capture the flux of solute through the boundary layer. These equations enable the determination of concentration profiles of solute across the thickness of the boundary layer (e.g., concentration polarization) to be calculated.

As a simple example, consider a traditional stirred cell experiment operating at pseudo-steady state. Traditionally, experiments are conducted at low solvent recoveries such that the feed concentration remains constant. The total flux of solute through the boundary layer is equal to the total flux of solute through the membrane. A common estimate of the solute flux is represented by Ji=Jwcp,i. Where cp,i is the concentration of species ‘i’ in the permeate. Substituting this estimate for Ji and numerically integrating the equation from the bulk solution to an arbitrary thickness provides the concentration of the ions throughout the boundary thickness.

| Jwcp,1s=c1sJw+∇c1sD11s+∇c2sD12s | Eq 23 |
| --- | --- |

| Jwcp,2s=c2sJw+∇c1sD21s+∇c2sD22s | Eq 24 |
| --- | --- |

| Jwcp,3s=c3sJw+∇c1sD31s+∇c2sD32s | Eq 25 |
| --- | --- |

- Membrane Phase Transport: General solute flux equation through the membrane.

The solute flux within the membrane is derived in a similar manner as above. The extended Nernst-Planck equation (equation 1), net-ionic current constraint (equation 2), and electroneutrality constraint (equation 3) remains. A new term, χ, represents the fixed charge on the membrane. In this derivation, χ is assumed positive. Following the assumptions above, the cations (z1, z2) share a common anion (z3). We assume that the membrane is homogenous, therefore χ remains constant throughout the membrane thickness, ∇χ=0.

| jim=cimJw-Dim∇cim-ziDimFRTcim∇ϕ | Eq 26 |
| --- | --- |

| I=0= Sinzijim dr | Eq 27 |
| --- | --- |

| inzicim=0 | Eq 28 |
| --- | --- |

| z1c1s+z2c2s+ z3c3s+χ=0 | Eq 29 |
| --- | --- |

| inzi∇cim=0 | Eq 30 |
| --- | --- |

| z1∇c1s+z2∇c2s+z3∇c3s=0 | Eq 31 |
| --- | --- |

As aforementioned, we solve for the electrical potential by setting equation 26 equal to equation 27.

| j1m= 0=z1c1mJw-z1D1m∇c1m-z12D1mFRTc1m∇ϕ+z2c2mJw-z2D2m∇c2m-z22D2mFRTc2m∇ϕ+z3c3mJw-z3D3m∇c3m+z32D3mFRTc3m∇ϕ | Eq 32 |
| --- | --- |

| 0=Jwz1c1m+z2c2m+z3c3m-[z1D1m∇c1m+z2D2m∇c2m+z3D3m∇c3m]-FRT∇ϕz12D1mc1m+z22D2mc2m+z32D3mc3m | Eq 33 |
| --- | --- |

| FRT∇ϕz12D1mc1m+z22D2mc2m+z32D3mc3m=Jwz1c1m+z2c2m+z3c3m-[z1D1m∇c1m+z2D2m∇c2m+z3D3m∇c3m] | Eq 34 |
| --- | --- |

| ∇ϕ=RTF[Jwz1c1m+z2c2m+z3c3m-[z1D1m∇c1m+z2D2m∇c2m+z3D3m∇c3m]z12D1mc1m+z22D2mc2m+z32D3mc3m] | Eq 35 |
| --- | --- |

We apply the electroneutrality constraint (equations 29, 31) to equation 35.

| ∇ϕ=RTF[Jw-χ-[z1D1m∇c1m+z2D2m∇c2m+D3m(-z1∇c1m-z2∇c2m)]z12D1mc1m+z22D2mc2m+z3D3m(-z1c1m-z2c2m-χ)] | Eq 36 |
| --- | --- |

After simplification,

| ∇ϕ=-RTF[Jwχ+∇c1mz1D1m-z1D3m+∇c2mz2D2m-z2D3m c1m z12D1m-z1z3D3m+ c2mz22D2m-z2z3D3m-z3χD3m] | Eq 37 |
| --- | --- |

We can now solve for the individual ion fluxes in the membrane phase. Taking equation 37 and substituting equation 26, we solve for the flux of cation 1 in the membrane.

| j1m=c1mJw-D1m∇c1m+z1D1mc1mJwχ+∇c1mz1D1m-z1D3m)+ ∇c2m(z2D2m-z2D3mc1mz12D1m-z1z3D3m+c2mz22D2m-z2z3D3m-z3χD3m | Eq 38 |
| --- | --- |

We aim to put the above membrane phase solute flux equation in the form j1m=c1mJw+D11m∇c1m+D12m∇c2m. Equation 38 is mathematically equivalent to equation 39.

| j1m= c1mJw-D1m∇c1mc1mz12D1m-z1z3D3m+c2mz22D2m-z2z3D3m-z3χD3mc1mz12D1m-z1z3D3m+c2mz22D2m-z2z3D3m-z3χD3m +z1D1mc1mJwχ+∇c1mz1D1m-z1D3m)+ ∇c2m(z2D2m-z2D3mc1mz12D1m-z1z3D3m+c2mz22D2m-z2z3D3m-z3χD3m | Eq 39 |
| --- | --- |

Simplifying to equation 40,

| j1m=Jwc1m1+z1χD1mc1mz12D1m-z1z3D3m+c2mz22D2m-z2z3D3m-z3χD3m +∇c1mc1mz1z3D1mD3m-z12D1mD3m+c2mz2z3D1mD3m-z22D1mD2m+z3χD1mD3mc1mz12D1m-z1z3D3m+c2mz22D2m-z2z3D3m-z3χD3m+∇c2mz1z2D1mc1m(D2m-D3m)c1mz12D1m-z1z3D3m+c2mz22D2m-z2z3D3m-z3χD3m | Eq 40 |
| --- | --- |

Where the diffusion terms, D11m and D12m, are represented in the parenthesis.

In the limit that χ=0, the equation collapses to the same form as equation 16 for the solution phase solute flux,

| j1m=Jwc1m +∇c1mc1mz1z3D1mD3m-z12D1mD3m+c2m(z2z3D1mD3m-z22D1mD2m)c1mz12D1m-z1z3D3m+c2mz22D2m-z2z3D3m+∇c2mz1z2D1mc1m(D2m-D3m)c1mz12D1m-z1z3D3m+c2mz22D2m-z2z3D3m | Eq 41 |
| --- | --- |

We repeat this process to solve for the ion flux of cation 2 in the form is j2m=c2mJw+D21m∇c1m+D22m∇c2m, equation 42,

| j2m=Jwc2m1+z2χD2mc1mz12D1m-z1z3D3m+c2mz22D2m-z2z3D3m-z3χD3m +∇c1mz1z2D2mc2m(D1m-D3m)c1mz12D1m-z1z3D3m+c2mz22D2m-z2z3D3m-z3χD3m+∇c2mc1mz1z3D2mD3m-z12D1mD2m+c2mz2z3D2mD3m-z22D2mD3m+z3χD2mD3mc1mz12D1m-z1z3D3m+c2mz22D2m-z2z3D3m-z3χD3m | Eq 42 |
| --- | --- |

Again, where the diffusion terms, D21m and D22m, are represented in the parenthesis.

For j3m=c3mJw+D31m∇c1m+D32m∇c2m, the above derivation can be repeated. Alternatively, the net-ionic current constraint can be used. Recall equation 27,

| I=0= Sinzijim dr | Eq 27 |
| --- | --- |

| z1j1m+z2j2m+z3j3m=0 | Eq 43 |
| --- | --- |

| j3s=-z1z3j1m-z2z3j2m | Eq 44 |
| --- | --- |

- Solute flux equation simplification (in the limit that diffusion, convection, or electromigration are negligible)

The derivation of the solute flux in both the solution and membrane phase can be repeated by assuming only certain transport terms are relevant for the system. The following table represents which transport mechanisms dominate in each model explored. The equations representing the relevant transport mechanisms can be derived following the step above, where negligible transport mechanisms are reduced to 0.

| Model | Diffusion | Convection | Electromigration |
| --- | --- | --- | --- |
| M1 | X | X | X |
| M2 | X | X |  |
| M3 | X |  | X |
| M4 |  | X | X |
| M5 | X |  |  |
| M6 |  | X |  |

- Derivation of theoretical thermodynamic equations for solution and membrane phase transport at the boundary layer.

We begin by equating the chemical potential of the ion, i, in the solution to that in the  membrane, μi0,m=μi0,s, where μi0 is the reference state, RTlnγici is the entropic term, and ziFϕ is the electro-static term.

| μi0,m+RTlnγimcim+ziFϕm=μi0,s+RTlnγiscis+ziFϕs | Eq 45 |
| --- | --- |

Assuming there are no ions in solution, zi goes to 0 and there is no electrostatic potential across the solution and membrane interface. In this manner, the partitioning of ions is determined by the relative reference states within solution and the membrane.

| δi=exp⁡μi0,s-μi0,mRT | Eq 46 |
| --- | --- |

However, the systems we are interested in have at least one salt present and therefore create an electrostatic potential across the solution-membrane interface. We solve for this difference,

| ϕm-ϕs=RTziFμi0,s-μi0,mRT+lnγiscisγimcim | Eq 47 |
| --- | --- |

Following the same naming convention as previously stated, the cations in this two-salt system are denoted with subscripts 1 and 2 while the common anion is identified as subscript 3. Equating the electrostatic force of each ion gives,

| RTz1Fμ10,s-μ10,mRT+lnγ1sc1sγ1mc1m=RTz2Fμ20,s-μ20,mRT+lnγ2sc2sγ2mc2m=-RTz3Fμ30,s-μ30,mRT+lnγ3sc3sγ3mc3m | Eq 48 |
| --- | --- |

Isolating one salt allows us to derive an expression for the partitioning of cation (1) and anion (3). A similar process can be repeated for the other salt present, since a common anion is shared.

| RTz1Fμ10,s-μ10,mRT+lnγ1sc1sγ1mc1m=-RTz3Fμ30,s-μ30,mRT+lnγ3sc3sγ3mc3m | Eq 49 |
| --- | --- |

| μ10,s-μ10,mz1RT+1z1lnγ1sc1sγ1mc1m=-μ30,s-μ30,mz3RT-1z3lnγ3sc3sγ3mc3m | Eq 50 |
| --- | --- |

| μ10,s-μ10,mz1RT+μ30,s-μ30,mz3RT=-1z1lnγ1sc1sγ1mc1m-1z3lnγ3sc3sγ3mc3m | Eq 51 |
| --- | --- |

Using log rules, lna-lnb=lnab

| μ10,s-μ10,mz1RT+μ30,s-μ30,mz3RT=lnγ3sc3sγ3mc3m-1z3γ1sc1sγ1mc1m-1z1 | Eq 52 |
| --- | --- |

| expμ10,s-μ10,mz1RT+μ30,s-μ30,mz3RT=γ3mc3mγ3sc3s1z3γ1mc1mγ1sc1s1z1 | Eq 53 |
| --- | --- |

Using exponent rules, em+n=emen

| expμ10,s-μ10,mz1RTexpμ30,s-μ30,mz3RT=γ3mc3mγ3sc3s1z3γ1mc1mγ1sc1s1z1 | Eq 54 |
| --- | --- |

And emn= emn

| expμ10,s-μ10,mRT1z1expμ30,s-μ30,mRT1z3=γ3mc3mγ3sc3s1z3γ1mc1mγ1sc1s1z1 | Eq 55 |
| --- | --- |

Recall and substitute equation 46,

| δ11z1δ31z3=γ3mγ3sc3mc3s1z3γ1mγ1sc1mc1s1z1 | Eq 56 |
| --- | --- |
| δ3γ3sγ3mc3sc3m1z3=1δ1γ1mγ1sc1mc1s1z1 | Eq 57 |

These general relationships hold up for all three ions.

| 1δ1γ1mγ1sc1mc1s1z1=δ3γ3sγ3mc3sc3m1z3 | Eq 58 |
| --- | --- |
| 1δ2γ2mγ2sc2mc2s1z2=δ3γ3sγ3mc3sc3m1z3 | Eq 59 |
| 1δ1γ1mγ1sc1mc1s1z1=δ2γ2sγ2mc2sc2m1z2 | Eq 60 |

First, isolating c1m and c2m.

| c1m=δ1γ1sc1sγ1mδ2γ2sγ2mc2sc2mz1z2 | Eq 61 |
| --- | --- |
| c2m=δ2γ2sc2sγ2mδ1γ1sγ1mc1sc1mz2z1 | Eq 62 |

Instead of relating the partition of one species to the shared anion, it may be helpful to relate it to the other, competing, cation. Using electroneutrality where χ is the fixed membrane charge and we select the negatively charged NF270 membrane.

For the membrane.

| z1c1m+z2c2m-z3c3m-χ=0 | Eq 66 |
| --- | --- |
| c3m= z1z3c1m+z2z3c2m-χz3 |  |

For the solution phase.

| z1c1s+z2c2s-z3c3s=0 | Eq 66 |
| --- | --- |
| c3s= z1z3c1s+z2z3c2s |  |

Now we have all the pieces to relate the membrane phase concentration of one species to the other cation.

| 1δ1γ1mγ1sc1mc1s1z1=δ3γ3sγ3mc3sc3m1z3 | Eq 67 |
| --- | --- |
| 1δ1γ1mγ1sc1mc1s1z1=δ3γ3sγ3mz1z3c1s+z2z3c2sz1z3c1m+z2z3c2m-χz31z3 | Eq 68 |

And now substituting for the membrane phase c2m.

| 1δ1γ1mγ1sc1mc1s1z1=δ3γ3sγ3mz1z3c1s+z2z3c2sz1z3c1m+z2z3δ2γ2sc2sγ2mδ1γ1sγ1mc1sc1mz2z1-χz31z3 | Eq 69 |
| --- | --- |

Now, c1m is only a function of the solution phase concentrations and the charge density of the membrane.

Assuming δ1=δ2=γ1s= γ1m=γ2s= γ2m=1

| c1mc1s1z1=z1z3c1s+z2z3c2sz1z3c1m+z2z3c2sc1sc1mz2z1-χz31z3 | Eq 70 |
| --- | --- |

For the simplest system where all ion valencies are|1|such as NaCl, KCl (Na+ = 1, K+ = 2, Cl- = 3),

| c1mc1s=c1s+c2sc1m+c2sc1sc1m-χ | Eq 71 |
| --- | --- |
| c1mc1sc1m+c2sc1sc1m-χ=c1s+c2s | Eq 72 |

Defining the partition coefficient

| H1≡ c1mc1s | Eq 73 |
| --- | --- |
| H1c1mc1sc1s+c2sc1sc1m-χ=c1s+c2s | Eq 74 |
| H1H1c1s+c2sH1-χ=c1s+c2s | Eq 75 |
| H12c1s+c2s-χH1-(c1s+c2s)=0 | Eq 76 |
| H12c1s-χH1+c2s-(c1s+c2s)=0 | Eq 77 |
| c1sc1s+c2sH12-χc1s+c2sH1+c2sc1s+c2s-1=0 | Eq 78 |
| c1sc1s+c2sH12-χc1s+c2sH1-c1sc1s+c2s=0 | Eq 79 |

For a NaCl, CaCl2 system where Na+ = 1, Ca2+ = 2, Cl- = 3

| c1mc1s=c1s+2c2sc1m+2c2sc1sc1m2-χ | Eq 80 |
| --- | --- |
| c1mc1sc1m+2c2sc1sc1m2-χ=c1s+2c2s | Eq 81 |

Repeating the steps performed in Eqs 73-77, we obtain

| H13c1sc1s+2c2s-χc1s+2c2sH12-H1+2c2sc1s+2c2s=0 | Eq 82 |
| --- | --- |

Repeating these steps for a NaCl, LaCl3 system (Na+ = 1, La3+ = 2, Cl- = 3),

| c1mc1s=c1s+3c2sc1m+3c2sc1sc1m3-χ | Eq 83 |
| --- | --- |
| c1mc1sc1m+3c2sc1sc1m3-χ=c1s+3c2s | Eq 84 |

Repeating the steps performed in Eqs 73-79, we obtain

| H14c1sc1s+3c2s-χc1s+3c2sH13-H12+3c2sc1s+3c2s=0 | Eq 85 |
| --- | --- |

And lastly, we repeat these steps for a CaCl2 and LaCl3 system (Ca2+ = 1, La3+ = 2, Cl- = 3),

| c1mc1s12=2c1s+3c2s2c1m+3c2sc1sc1m3-χ | Eq 86 |
| --- | --- |
| c1mc1s=2c1s+3c2s2c1m+3c2sc1sc1m3-χ2 | Eq 87 |
| c1mc1s2c1m+3c2sc1sc1m3-χ2=2c1s+3c2s2 | Eq 88 |
| H14(c1m)2+12c1mc2sH13-4c1mχ+9(c2s)2(H13)2-6c2sχ(H13)2+χ2=4(c1s)2+12c1sc2s+9(c2s)2 | Eq 89 |

Multiply each term within the parenthesis by c1sc1s to get rid of the c1m,

| H14(c1m)2c1sc1s2+12c1mc2sH13c1sc1s-4c1mχc1sc1s+9(c2s)2(H13)2-6c2sχ(H13)2+χ2=4(c1s)2+12c1sc2s+9(c2s)2 | Eq 90 |
| --- | --- |
| H14H12c1s2+12c1sc2sH12-4c1sχH1+9(c2s)2H16-6c2sχH16+χ2=4(c1s)2+12c1sc2s+9(c2s)2 | Eq 91 |
| H14H14c1s2+12c1sc2s-4c1sχH13+9(c2s)2H14-6c2sχH14+χ2H12=H124(c1s)2+12c1sc2s+9(c2s)2 | Eq 92 |
| 4H15c1s2+12c1sc2sH1-4c1sχH14+c2s(9c2s-6χ)H13+χ2H13=H124(c1s)2+12c1sc2s+9(c2s)2 | Eq 93 |
| 4H18c1s2+12c1sc2sH14-4c1sχH17+c2s(9c2s-6χ)+χ2H16=H154(c1s)2+12c1sc2s+9(c2s)2 | Eq 94 |
| 4H18c1s2-4c1sχH17+χ2H16-H154c1s2+12c1sc2s+9(c2s)2+12c1sc2sH14+c2s9c2s-6χ=0 | Eq 95 |

Divide each term by 4c1s2,

| H18-χc1sH17+χ24c1s2H16-4c1s2+12c1sc2s+9(c2s)24c1s2H15+3c2sc1sH14+c2s9c2s-6χ4c1s2=0 | Eq 96 |
| --- | --- |

Below are the compiled equations where the highest valency H1 term is isolated for:

NaCl + KCl (1:1:1)

| c1sc1s+c2sH12-χc1s+c2sH1-c1sc1s+c2s=0 | Eq 79 |
| --- | --- |
| H12-χc1sH1-1=0 |  |

NaCl + CaCl2 (1:2:1)

| H13c1sc1s+2c2s-χc1s+2c2sH12-H1+2c2sc1s+2c2s=0 | Eq 82 |
| --- | --- |
| H13-χc1sH12-1+2c2sc1sH1+2c2sc1s=0 |  |

NaCl + LaCl3 (1:3:1)

| H14c1sc1s+3c2s-χc1s+3c2sH13-H12+3c2sc1s+3c2s=0 | Eq 85 |
| --- | --- |
| H14-χc1sH13-1+3c2sc1sH12+3c2sc1s=0 |  |

CaCl2 + LaCl3 (2:3:1)

| 4H18c1s2+12c1sc2sH14-4c1sχH17+χ2H16-H154c1s2+12c1sc2s+9(c2s)2+c2s9c2s-6χ=0 | Eq 95 |
| --- | --- |
| 4c1s2c2s9c2s-6χH18-4c1sχc2s9c2s-6χH17+χ2c2s9c2s-6χH16-4c1s2+12c1sc2s+9(c2s)2c2s9c2s-6χH15+4c1sc2sc2s3c2s-2χH14+1=0 |  |
| H18-χc1sH17+χ24c1s2H16-4c1s2+12c1sc2s+9(c2s)24c1s2H15+3c2sc1sH14+c2s9c2s-6χ4c1s2=0 | Eq 96 |

Section 2.1: Bulk solution (numbers 1 ,2, and 4)

The bulk solutions are assumed to be well-mixed.

cisγis=ais

Where γis is equal to 1.

Section 2.2: Membrane (number 3)

In the membrane, you can have manning counter-ion condensation, ion association, etc. Nevertheless, we assume that these effects are negligible and assume that γim=1.

cimγim=aim

Section 3: Summary of equations in “usable”/generalized form.