# PROMPT — Generalize the DATA2 time correction + DATA2-styled prediction plots

You are working in the dynamic-diafiltration-pyomo repo. Three deliverables, all in refactored_codes_v1/. Read before you write: skim the files referenced in each task and confirm the actual function names / line numbers (they drift) before editing.

## Repository orientation

- Core library: refactored_codes_v1/refactored_ucb_library.py

  - Excel/legacy loader: _load_legacy_data_stru_from_excel(...) (~line 4780) — builds data_stru.

  - Model solve: solve_model(...) → returns (fit, sim, ...).

  - Existing comparison plots: plot_nf270_data_comparison(...) (~13486) and plot_data_comparison(...) (~5631).

  - Permeate time-correction metadata already lives on data_stru["permeate_time_correction"].

- Existing prediction-plot generator: refactored_codes_v1/_make_predictions.py.

- Conductivity model: refactored_codes_v1/conductivity_paper.py (variant_shedlovsky(...) → single-salt specific conductivity in mS/cm; msa_transport(...) → multi-salt) and refactored_codes_v1/CONDUCTIVITY_PAPER_EXPLAINER.md.

- DATA2 .mat time-correction reference: UnifiedFramework/Trials/Data_loader/load_data.m (MATLAB — see eqns around lines 215–233 for the vial-close / shared-axis convention).

- Python envs: ~/miniforge3/envs/my-idaes-env/bin/python (pyomo + IPOPT, for solving) and ~/miniforge3/bin/python (plotting). Plotting tasks do not need IPOPT.

## Hard constraints (do not violate)

- Do NOT modify any MATLAB .m file. load_data.m even has unresolved merge-conflict markers — read it for the convention only; never edit it.

- Do NOT change DATA1 or DATA2 behavior. All new behavior must be opt-in / config-driven so existing campaigns are byte-for-byte unaffected.

- Match the DATA2 plotting convention (colors + markers) — do not invent a palette.

- Verify, don't assume: keep the DATA1/DATA2 pytest suite green and visually inspect every new/changed figure.

## Task 1 — Make the DATA2 time correction a reusable, campaign-level feature

Problem. The DATA2 .mat workflow applies a time adjustment to the mass, retentate, and permeate streams (shared time axis; the single ICP permeate value compared at vial close, m.cV[vial, tau.last()]). DATA3/NF270 needs the same correction, but it currently has to be re-applied/patched by hand each time.

Do this. Implement the DATA2 time-correction logic as a single reusable routine in refactored_ucb_library.py (e.g. apply_campaign_time_correction(data_stru, convention=...), or a config flag honored inside _load_legacy_data_stru_from_excel), driven by per-campaign configuration so any campaign can enable it declaratively — DATA3 turned on now, others later — with no manual editing per run.

- Source the convention from load_data.m (read-only) and from the existing DATA3 vial-close work already in the loader; consolidate them into one code path.

- Record what was applied on data_stru metadata (method, anchor, which streams), as is done today for permeate_time_correction.

Acceptance.

- A DATA3 sheet loads with the correction applied automatically via config (no hand-patching).

- DATA1 and DATA2 loads are unchanged (the correction is off / identical for them).

- python -m pytest pytest_refactored_codes_v1/tests/test_data1_smoke.py pytest_refactored_codes_v1/tests/test_data2_paper_comparison.py -q stays green.

- A short note added to Architecture.md describing the new config knob.

## Task 2 — Plot-cleanup utility (legend never covers the data; DATA2 colors/symbols)

Problem. In the mass-vs-time and concentration-vs-time comparison plots the legend overlaps the trends being analyzed.

Do this. Add a small, reusable styling/cleanup helper (new module refactored_codes_v1/_plot_style.py, or a helper in the library) and apply it to the existing mass-vs-time and concentration-vs-time plots so:

- The legend is moved off the data — place it outside the axes (e.g. bbox_to_anchor) or auto-select the emptiest quadrant; never occlude the curves.

- A single shared DATA2_STYLE dict defines colors + markers for: retentate (measurements), permeate (measurements), vial (measurements), retentate ICP, permeate/retentate predictions (model lines). Extract these exact colors/markers from the DATA2 plotting code (plot_data_comparison and/or the DATA2 reproduction notebook/figures) — do not guess.

- Reuse the same DATA2_STYLE everywhere (Task 2 and Task 3) for consistency.

Acceptance. Regenerate a representative mass and concentration plot; confirm by rendering to PNG that the legend sits outside/clear of the data and colors/markers match DATA2.

## Task 3 — New per-experiment prediction-fit figure set (5 quantities)

For each experiment (per sheet/run), produce a set of time-series figures comparing model/prediction to data, reusing DATA2_STYLE and the Task-2 legend handling:

- Mass per vial vs time — keep the existing plot as-is (it's good).

- ICP concentrations vs time — retentate and permeate ICP points (separate figure).

- Conductivity-probe signal vs time — raw probe trace for retentate and permeate (separate figure). Find the raw conductivity time series in the loader/data_stru.

- Conductivity-derived concentration vs time — convert the probe conductivity to concentration using conductivity_paper.py (invert variant_shedlovsky per point with a root-find such as scipy.optimize.bisect, using the salt-specific parameters), plotted for retentate and permeate (separate figure). Note in code how this compares to the loader's existing conductivity→concentration conversion.

- Applied pressure (ΔP) vs time — separate figure.

Implementation notes:

- Build this as a new generator (e.g. _make_prediction_panels.py) or extend _make_predictions.py; keep it driven by the same run registry used elsewhere.

- One figure per quantity per experiment (separate files), saved under the predictions output dir; also emit a combined multi-panel overview per experiment if cheap.

- Single-salt inversion is sufficient for the current DATA3 campaign; leave a clear hook for multi-salt via msa_transport if needed later.

Acceptance. Render the 5 figures for one representative DATA3 sheet (e.g. a concentrating CaCl₂ run); verify each is readable, legend-clear, DATA2-styled, and that the conductivity-derived concentration tracks the ICP points sensibly.

## Task 4 — Per-sheet concentration-range table

For each experimental data sheet, generate a table summarizing the range (min–max) of the key concentrations [mM] over the run:

- Diafiltrate c_D

- Feed-side retentate c_F

- Interfacial c_in (membrane surface)

- Permeate c_H

Notes:

- c_D, c_F, c_H come from the loaded data (data_stru / ICP / conductivity-derived series); c_in is a model quantity — take it from the fitted forward-sim (solve_model(...) → sim[vial]["cIn"]). Label each column with its source (measured vs model).

- Report min and max per stream (mean optional); keep units consistent (mM).

- Emit one table per sheet AND a combined campaign summary (one row per sheet) as CSV and a human-readable markdown table.

Acceptance. A range table is produced for every DATA3 sheet; spot-check that c_F spans the dilution/concentration trajectory and that c_in sits on the expected side of c_F given the polarization direction (concentrating: c_in > c_F; diluting: c_in < c_F).

## Task 5 — WSSE identifiability contours for the new response channels

Extend the existing per-response WSSE contour analysis (σ×B log-WSSE surfaces at fixed Lₚ — the same method and visual style as the current mass / retentate / permeate contours) to the new response channels from Task 3. Produce one contour per channel:

- Mass per vial (as today).

- ICP retentate + permeate concentration.

- Conductivity-probe signal (retentate + permeate) — WSSE computed in conductivity space: map the model concentration → predicted conductivity via conductivity_paper.variant_shedlovsky(...) and compare to the measured probe trace.

- Conductivity-derived concentration (retentate + permeate) — WSSE computed in concentration space: invert the probe conductivity → concentration (Task 3 inversion) and compare to the model concentration.

Implementation:

- Reuse the existing contour machinery — _run_Bsigma_fixedLp.py, calc_contour_2d_py / calc_contour_3d_py, calc_FIM, and the per-response WSSE objective branches in refactored_ucb_library.py — plus the established contour styling (the "v7" look with the optimum-marker box, cf. _replot_Bsigma_titled.py / _rerender_contours_with_optimum_box.py).

- The conductivity channels add two new residual definitions (conductivity-space and concentration-space); wire them in as additional per-response objective channels without altering the existing mass/retentate/permeate channels.

- ΔP (applied pressure) is a control input, not a fitted response — no WSSE contour for it.

Acceptance. A log-WSSE σ×B contour rendered per response channel for a representative DATA3 sheet, in the same style as the existing contours, with the optimum marker shown; confirm the per-channel identifiability pattern (narrow valley in B, ~flat in σ) is consistent with the established single-experiment finding.

## Task 6 — Rejection vs log(Pe) diagnostic (σ-family) for choosing σ per experiment (analysis phase)

Build an analysis-phase plot of rejection R (y-axis) vs log(Pe) (x-axis), overlaying one trajectory per σ value so the best σ to use for a given experiment can be read off the figure.

- Each trajectory is the Spiegler–Kedem rejection curve at a fixed σ: R(Pe; σ) = σ(1 − e^−Pe) / (1 − σ·e^−Pe), swept over log(Pe) for a grid of σ (e.g. 0.1 … 0.99). Each curve rises from R → 0 at Pe → 0 to a plateau R → σ at Pe ≫ 1.

- Overlay the experiment's observed/fitted rejection at its operating Pe (use Pe / Pe* from the Péclet analysis); the σ-curve passing through that point is the best-supported σ — mark it.

- Purpose: a visual, analysis-phase way to select σ per experiment and to see how tightly σ is constrained (ties into the established single-experiment σ–B identifiability finding — when the operating Pe is large, many σ-curves bunch near their plateau, so σ is poorly distinguished).

- Reuse the Pe definition/sweep from Tasks 1/5; use the established plot styling + the Task-2 legend handling (legend off the data; a labeled σ per curve).

Acceptance. An R-vs-log(Pe) figure with the σ-family of curves overlaid for a representative DATA3 sheet, the experiment's operating point marked, and the best-fit σ identifiable.

## Task 7 — Fix the diafiltrate feed rates (S0, S) from recorded values for DATA3 (lag mode)

Context. All DATA3/NF270 runs are lag mode. In lag mode the cell mass balance (ode_mF_rule, ~line 2029 of refactored_ucb_library.py) is dmF/dt = −S0 − area·ρ·Jw on the active concentration legs and dmF/dt = S/3600 on the final closed leg. Today S0 is a fixed Param but seeded from an M_O-based guess, and S is a free Var seeded from M_O/Σ(tf−ti)·3600 (~lines 1810, 1956–1958). So the apparatus feed terms are guessed/estimated rather than taken from the experiment — even though these flow rates are recorded per run. They should be fixed inputs, not fitted.

Solution (implement this):

- Ingest the recorded feed rates S0 (g/s) and S (g/hr) per experiment into data_config (from the run logs / Excel) under explicit keys, e.g. feed_rate_S0_g_per_s, feed_rate_S_g_per_hr.

- In model_construct_inter, for the DATA3 lag path only, set BOTH m.S0 and m.S as fixed Params from those data_config values — replace the M_O-based seed for S0 and the m.S = Var(...) for S (~lines 1950–1958). Guard with workflow_family == "DATA3"; leave the DATA1/DATA2 and Overflow branches byte-identical.

- Fall back to the current behavior if a recorded value is missing (don't break loading), and record on data_stru metadata which source was used (recorded vs fallback).

Why. Removes the only free apparatus degree of freedom (S), so the fit estimates just the transport parameters of interest (Lₚ, the B(c) coefficients, σ). It also eliminates a phantom free variable on any run with no final closed leg (where S never enters the balance and would otherwise drift unconstrained).

Caveat. This is a correctness / parameter-count cleanup, not an σ-identifiability fix — σ stays flat for the transport-regime reason (high-Pe plateau). Expect fitted Lₚ/B/σ to be ~unchanged (S was already pinned by the mass channel), just with one fewer free parameter.

Acceptance. A DATA3 lag sheet fits with S0/S fixed from recorded values (metadata shows the source); the free-parameter count drops by one; DATA1/DATA2 results are byte-identical; pytest green.

## Task 8 — Square (forward-solve) WSSE contour surfaces for every B(c) form × every DATA3 single-salt experiment

Objective. For each DATA3 single-salt experiment (all sheets) and each B(c) correlation — constant / linear / quadratic / cubic / saturating-exp / donnan — generate the WSSE contour surface for the response channels of Task 5, computed as a square forward solve at every grid node (no per-node nonlinear optimization).

How every node stays square (the decision rule):

- Fit each B-form once per experiment (or read its result_<form>.json) → the best-fit (maximum-likelihood) parameter set θ*.

- Hold fixed across the whole grid: Lp (its optimum), S0 and S (recorded feed rates, per Task 7), and every B(c) coefficient that is NOT on a contour axis, pinned at θ*.

- Axes = σ × [one B parameter] (e.g. the leading coefficient / in-window B level). With two axes setting two unknowns and everything else pinned, the model has zero free parameters → forward-solve the DAE (square) and evaluate the WSSE at each node.

- This is the slice convention — a conditional landscape through the optimum (its minimum coincides with the fitted (σ, B)). Label it as conditional, not a profile.

Response channels (per Task 5), one contour panel each: mass; ICP retentate conc; ICP permeate conc; conductivity-probe (retentate + permeate, conductivity space); conductivity-derived concentration (retentate + permeate, concentration space). ΔP is excluded (control input).

Implementation.

- Reuse calc_contour_2d_py / the method="slice" path of _run_profile_contours.py plus the established contour styling (optimum marker); extend the channel set to the conductivity ones.

- Every node is a CPU-capped forward solve (ideally with a bounded CasADi/IDAS init — integrator_options={'max_num_steps': …} — or skip_sim_init), so it runs fast and avoids the per-node optimization holes the profile campaign hit.

- Optional robustness check: regenerate a slice at the MLE ± standard error of the pinned coefficients to confirm the conclusion is insensitive to the pin.

Deliverable. One multi-panel contour figure per (experiment × B-form) — a panel per response channel — covering all DATA3 single-salt sheets × all six forms, saved under bform_study/ with a small index.

Acceptance. Square forward-solve contours produced for every (experiment, B-form, channel); spot-check that each surface's minimum sits at the fitted (σ, B) and that σ is ~flat (the expected non-identifiability), consistent across forms and salts.

## Task 9 — Profile-likelihood WSSE contour surfaces (rigorous counterpart of Task 8)

Objective. Same scope as Task 8 — every DATA3 single-salt experiment × every B(c) form × the Task-5 response channels — but compute the profile-likelihood surface: at each grid node, do NOT pre-fix the non-axis coefficients — re-optimize them.

Method (per node). Clamp the two axes (σ and the chosen B parameter), hold Lp, S0, S fixed (Lp optimum; S0/S recorded, per Task 7), then solve a sub-optimization over the remaining free parameters λ (the non-axis B coefficients):

Phi_profile(psi)  =  min over lambda  of  WSSE(psi, lambda | Lp, S0, S fixed)

Record Phi_profile as the pixel value, AND store lambda*(psi) (the profiled nuisance values).

Why (vs Task 8). Task 8 pins λ at the MLE → a conditional (slice) surface, which is not a valid confidence region. The profile re-optimizes λ → the surface supports likelihood-ratio confidence regions:

CR  =  { psi :  Phi_profile(psi) - Phi_min  <=  chi-square threshold }

Use it when the goal is honest identifiability / confidence intervals on σ and the B parameters.

By-product to plot. The profiled trajectories lambda*(psi) (e.g. β₀ as σ varies) directly visualize the parameter correlations (the β₀–β₁ trade-off; whether β₁ tracks σ).

Cost & machinery (NOT square). This is a nonlinear optimization at every node — you cannot keep it a forward solve. Reuse the existing _run_profile_contours.py method="profile" campaign:

- warm-start continuation (seed each node from a converged neighbour — a guess, not a fix),

- per-node IPOPT CPU cap + a bounded / skip_sim_init CasADi init,

- watchdog kill-restart + JSONL resume for stiff nodes (expect occasional holes). Task 7 (fixed S0/S) shrinks λ, so each per-node solve is faster and more robust.

Deliverable. One multi-panel profile contour per (experiment × B-form) — a panel per response channel — with the LR confidence regions overlaid (optionally the profiled-λ trajectories); all DATA3 single-salt sheets × all six forms.

Acceptance. Profile surfaces produced for every (experiment, B-form, channel); each minimum coincides with the fit; LR confidence regions drawn; the flat-σ non-identifiability is confirmed rigorously (not just conditionally), and any difference from the Task-8 slice is noted.

## Verification checklist (run at the end)

- pytest DATA1 + DATA2 smoke/paper-comparison green (DATA1/DATA2 untouched).

- Per-sheet concentration-range table (Task 4) emitted as CSV + markdown for each DATA3 sheet.

- WSSE σ×B contours (Task 5) rendered per response channel for a representative sheet, matching the established contour style.

- Rejection-vs-log(Pe) σ-family diagnostic (Task 6) rendered for a representative sheet, with the experiment's operating point marked and best-fit σ identifiable.

- Diafiltrate feed rates S0/S fixed from recorded values for DATA3 lag fits (Task 7); free-param count reduced by one; DATA1/DATA2 byte-identical.

- Square forward-solve WSSE contour surfaces (Task 8) generated for every DATA3 single-salt experiment × every B(c) form × every Task-5 response channel, with non-axis coefficients pinned at the per-form MLE; each surface's minimum coincides with the fit.

- Profile-likelihood WSSE contour surfaces (Task 9) generated per experiment × B-form × channel with non-axis coefficients re-optimized per node; LR confidence regions drawn; flat-σ confirmed rigorously vs the Task-8 slice.

- A DATA3 sheet loads with the time correction on via config; metadata recorded.

- All regenerated/new figures rendered to PNG and visually inspected (legend clear of data, DATA2 colors/markers).

- Architecture.md updated with the new time-correction config + plotting helpers.

## Assumptions baked in (change if wrong)

- Task 3 figures are separate files per quantity per experiment (+ optional combined panel).

- Conductivity→concentration uses single-salt variant_shedlovsky inversion for now.

- "Concentration vs time" plots cover both retentate and permeate on shared axes where it aids comparison; split if it gets crowded.